# MPM-toolbox

## AIMS

MPM-functions to run the hMRI-toolbox for rodent data (mouse/rat), i.e. for quantitative MRI.

For hMRI-toolbox see

https://www.cbs.mpg.de/abteilungen/neurophysik/software/hmri-toolbox

I) Working with Preprocessing steps .. Working with ANTx

## Preprocessing using ANTx2

-start Matlab

-add ANTx-path via hyperlink or go to ANTx-folder and type “antlink”  (this will set the paths of ANTx2)

-add MPM-path via hyperlink or go to mpm-folder and type “mpmlink”  (this will set the paths of mpm)

Set Matlab’s current working dir to the studies folder

My current study-folder is F:\data8_MPM\MPM_agBrandt3. This folder contains the subfolder ‘raw’ with two raw data-sets (Bruker format).

Type “ant” in Matlab command window to open the ANT-gui.

Now, create a new project by selecting Main/New project.

Here you have to specify the reference template. For this, select green icon left to “x.wa.refpath”:

From the template GUI/left listbox select the “mouse Allen2017HikishimaLR”-template. Hit [OK]-button.

Next, set the following parameters:

x.wa.usePriorskullstrip   = 3

x.wa.orientType              =  11

The parameter settings should know look as follows:

These two parameters .## REASON ##

Hit [OK]-button.

When asked, save the project-file for instance as “proj.m” in the study-folder (here the study folder is F:\data8_MPM\MPM_agBrandt3). When asked to load the project-file (“proj.m”) click [yes]-button.

Now the project-file is defined and loaded into ANTx2-GUI.

IMPORT BRUKER DATA

We next have to import the Bruker data from the raw-folder.

For this click MAIN/Import Bruker data. From the ANT-GUI.

In the Bruker-Import GUI select the “raw”-folder from the right listbox. If selected, this folder will disappear and reappear in the lower listbox. Hit [Done]-button.

The next GUI displays all available files found in in the “raw”-folder

Here we have two datasets (see column “set”). And we select the turborare-file and the PD, T1 and MT for each data set.

Hit [OK]-button to import these files.

Next the Parameter-file for the Bruker-import appears:

Do nothing here. Hit [RUN]-button

Now, data from two animals were imported (“20220725AB_MPM_12-4” & “20220725AB_MPM_18-9”) as seen in the left listbox (ANTx GUI.)

The data of each animal is stored in the “dat”-folde in the current study-folder:

And each animal-folder contains the turborare, the PD, the T1 and MT in NIFTI-format (see fig below):

Starting MPM-GUI

Set the path of the MPM-wrapper functions now, if not done before (see above).

Type “mpm” in Matlab’s command window to start the mpm-GUI

Configure mpm-setup and parameter files

Next hit the [setup]-button (this has to be done only once per study).

IN the mpm_setup GUI, check that all “copy”-radios are checked. Next, select the destination path by clicking the [select]-button. Here select the current study-folder (my folder is “MPM_agBrandt3”). In most cases an “mpm”-folder is added to the path. Next hit [copy files] to cype these three files to the new “mpm”-folder within the current study folder. When done hit [close]-button to close the mpm_setup GUI.

The new “mpm”-folder within the current study folder will now contain 3 files, which have to be modified to run the MPM-wrapper functions. The names of the three files are fixed:

1) “mpm_NIFTIparameters.xlsx”

2) “mpm_config.m”

3) “hmri_local_defaults_mouse.m”

1) “mpm_NIFTIparameters.xlsx”

The excelfile looks as follows:

Please enter the original NIFTI-filenames in the 2 column (original Filename). For the current study I inserted the names of:

-turborare (‘03_1_T2_TurboRARE_1.nii’)

-T1 (‘04_2_MPM_3D_0p15_t1_exvivo_1.nii‘)

-MT (‘04_3_MPM_3D_0p15_mt_exvivo_1.nii’) and

-PD (‘04_1_MPM_3D_0p15_pd_exvivo_1.nii’).

The correctness of the NIFTI-filenames in column-2 is important.  The excelfile will later contain important parameters needed for the hMRI-TBX (extracted from the Bruker raw data).

Please close the excel-document when finished, because parameters will be successively written into this file.

2) “mpm_config.m”

The mpm_config.m file contains parameters that have to be adjusted:

mpm.MPM_path      ='F:\data7\MPM_mouse\hMRI-toolbox-0.2.4'   ;%MPM-toolbox path

-please specify the path of the hMRI-toolbox-0.2.4

mpm.SPM_path      ='F:\data7\MPM_mouse\spm12'                ;%SPM-toolbox path

-please specify the path of SPM12.

-If SPM12 from the ANTx2-toolbox is used, keep this field empty (mpm.SPM_path     ='';)

The mpm.hrmi_defaults   and mpm.NIFTI_parameters should be already defined (after the mpm-setup), and link to the files “hmri_local_defaults_mouse.m” and “mpm_NIFTIparameters.xlsx”:

mpm.hrmi_defaults         ='F:\data8_MPM\MPM_agBrandt3\mpm\hmri_local_defaults_mouse.m';         %path of hMRI-configfile (m-file)

mpm.NIFTI_parameters ='F:\data8_MPM\MPM_agBrandt3\mpm\mpm_NIFTIparameters.xlsx';         %path of NIFTI-parameter settings (excelfile)

The mpm.t2w_preorient variable defines the rough orientation of the turborare-image (used for registration) to match the orientation of PD/T1/MT.

mpm.t2w_preorient=[1.5708 0 1.5708];  % preorientation (rotations) of turborare (t2) to match orientation with PD/T1/MT-images

The mpm.multifactor   is set to 2000. This factor is multiplied to the PD/T1/MT-image values to increase the dynamic range.mpm.multifactor           = 2000;   % multiply images by this factor to widen dynamic range

PD-normalization

For normalization of the PD-image an reference image is necessary that represents a mask of the CSF/ventricles. A ventricle mask exist in the mpm-toolbox and the path is defined in mpm.PD_normalizeMask.

mpm.PD_normalizeMask      ='F:\mpm\resources\mask_ventricle.nii';         %reference-mask for normalization ('mask_ventricle.nii' or

'mask_water.nii' or use your own mask)

mpm.PD_normalizeFunction  =@mean                                       ;% function to aggregate the values within reference-mask

mpm.PD_normalizeOutputName='PD_normalized.nii'                         ;% resulting outputName

The mpm.PD_normalizeFunction defines how to aggregate the values within the reference mask (here as mean over values within the mask). The output name of the normalized PD-file is defined in mpm.PD_normalizeOutputName.

3) “hmri_local_defaults_mouse.m”

This m-file contains parameters for the hmri-toolbox.

Important here is that the parameter hmri_def.TPM links to the path of the tissue compartments (contained in the mpm-toolbox):

hmri_def.TPM='F:\mpm\resources\mouseTPM_mod.nii';

LOAD AND RUN THE MPM-PIPELINE

First select those animals from the ANTx-toolbox (left listbox) that should be processed. Here I selected all animals.

If the mpm-GUI is not open, type “mpm” to open the mpm-GUI.

Now, load the mpm-configfile by clicking the [load config-file]-button, and select the “mpm_configfile.m” from the studies mpm-folder.

Hit [open/OK](“öffnen”)-button.

When done, the green message right to the [load config-file]-button should display the mpm_config-file

Select all tasks from the preproc-pipeline (left) and the hMRI-pipeline (right).

Now, hit [RUN] from the mpm-GUI to start processing for all animals selected from the ANTx-GUI.

When processing is finished, each animal folder will contain a PD-normalized image “PD_normalized.nii” and a subfolder (“Results”) with resulting NIFTI-files.

PREPROC-Processing steps (ANTx is mandatory for the following preprocessing steps)

All steps must be processed sequentially in that order

1) get BrukerParams: This step is done after importing the Bruker raw-data and specifying the NIFTI-files in column-2 of the excelfile (‘mpm_NIFTIparameters.xlsx’). Based on the Bruker-import and the filenames in the excelfile (‘mpm_NIFTIparameters.xlsx’), the Parameters  flip-angle, TR and TE for T1, MT and PD are extracted and written in the respective columns of the excelfile (columns: FA_deg, TR_ms,TE_ms).

2) rename files: In the next step, the NIFTI-files (with names defined in column-2 of the excel-file) are copied and the copies renamed to fixed names, defined in column-1 of the excelfile (‘mpm_NIFTIparameters.xlsx’).

Specifically, local copies of the following files are created and new names are given to the copies of the following files:

'03_1_T2_TurboRARE_1.nii'  --> 't2w.nii'

'04_2_MPM_3D_0p15_t1_exvivo_1.nii' -->'T1.nii'

'MT	04_3_MPM_3D_0p15_mt_exvivo_1.nii'-->'MT.nii'

'PD	04_1_MPM_3D_0p15_pd_exvivo_1.nii'-->'PD.nii'

3) register turborare to T1/PD/MT: In this step the, the turborare-image is rigidly registered to T1/PD/MT.

Here it is assumed that T1/PD/MT are in register towards each other. On the other hand, T1/PD/MT  are not in register with the turborare image ('t2w.nii') which is later used for registration with the reference/template (here Allen mouse brain, here we call the space standard-space). Thus, 't2w.nii' must be registered to T1/PD/MT.

If the orientation is completely different between 't2w.nii' and T1/PD/MT there is the option to add a pre-orientation (3 rotation angles). This pre-orientation has to be defined in the mpm.t2w_preorient-variable (for this study it is:  [1.5708 0 1.5708] in the ‘mpm_config.m’-file.

The resulting output file will be in register with T1/PD/MT and is named is ‘t2.nii’.

4) register to SS: Next, the ‘t2.nii’-image (i.e. the aligned turborare image) is registered to standard space (SS; i.e. the Allen mouse brain template/atlas).  When done, a new file is created (‘x_t2.nii’) which represents the ‘t2.nii’-image in standard space.

5) transform  to SS: Based on the previous step, the estimated transformation-parameters (rigid+affine+nonlinar)  are used to transform the following files from native space to standard space (SS):

‘T1.nii‘, ‘MT.nii‘ , ‘PD.nii‘ to ‘x_T1.nii‘, ‘x_MT.nii‘ , ‘x_PD.nii‘, respectively.

Aside this, the previous step (here SPM’s Unified Approach is used) created a biasfield-corrected ‘t2.nii’-image (‘mt2.nii’) which is also transformed to standard-space (‘x_mt2.nii’)

hMRI-Processing steps (with/without ANTx)

6) split 4D files: Here the 4D-volumes ‘x_T1.nii‘, ‘x_MT.nii‘ , ‘x_PD.nii‘ are split into 3D-volumes (x_T1_00001.nii, x_T1_00002.nii…etc.;    x_MT1_00001.nii, x_MT1_00002.nii…etc.; x_PD1_00001.nii, x_PD1_00002.nii…etc.)

7) add HDR-description: The parameters flip-angle, TR and TE for T1, MT and PD (which were previously written to the Excelfile (‘mpm_NIFTIparameters.xlsx’, see get BrukerParams ) are written to the header of x_T1,x_MT and x_PD images.

8) multiply by factor: In the next step a factor  is multiplied to the values of the of x_T1,x_MT and x_PD images to increase the dynamic range. Note that the factor is defined in the “mpm_config.m”-file (mpm.multifactor = 2000).

The factor is also stored in the header of the images to allow to rerun this step and prevent multiple, sequential multiplications of the same image with several factors.

9) run Model (estimate): Currently there is only one hMRI-implementation available 'f_runMRM_mode1.m'. Hit the info-button for more information.

Basically all images have to be in standard space (SS). The following inputs are used:

[RF]: 'x_t2.nii' and 'x_mt2.nii'

'x_t2.nii'     - is the t2-weighted image in SS

'x_mt2.nii' - is the bias-corrected t2-weighted image in SS ('mt2.nii'  is generated using SPM's Unified Approach)

[B1-Bias correction]: #r - B1-bias correction is not used here. I.e. the input is 'noB1'

[raw_mpm]:  PD, MT and T1 data in SS are used,  i.e. multiple 3D-volumes for each modality is used:

'x_PD_00001.nii'   ,   'x_PD_00002.nii'  ,  'x_PD_00003.nii'  ...etc.

'x_MT_00001.nii'  ,   'x_MT_00002.nii' ,  'x_MT_00003.nii'  ...etc.

'x_T1_00001.nii'   ,   'x_T1_00002.nii'   ,  'x_T1_00003.nii'   ...etc.

10) normalize: This step creates a normalizes PD-image ‘PD_normalized.nii’. For this step, a reference-mask has to be provided. The path of the reference mask (NIFTI-file) is specified in mpm_config.m-file (variable: ‘mpm.PD_normalizeMask’). The ventricle-mask ('mask_ventricle.nii', located in the mpm-tbx) is configured as the default reference mask.

Note: If another mask is used as reference mask, it must have the following properties:

-binary 3D NIFTI-image, with values equal to  ‘1’ referring to reference-regions such as ventricles

-mask must be in standard space (SS), i.e. in register with the template, x_t2.nii etc.

-same voxel-size as x, _t2.nii, x_T1,x_MT and x_PD images.

II) Working without the Preprocessing steps/Working without ANTx

The following workflow works without ANTx-TBX and can be used when the following files exist in standard space:

x_MT.nii    : 4D MT-volume in standard-space

x_T1.nii     : 4D T1-volume in standard-space

x_PD.nii    : 4D PD-volume in standard-space

x_t2.nii     : turborare in standard-space

x_mt2.nii : bias-corrected turborare in standard-space (created for example using SPM’s unified Approach)

For example: The following study-folder “F:\data8_MPM\MPM_agBrandt4_noANTx” contains a ‘dat’-folder with the animal-folders: ‘20220725AB_MPM_12-4’ and ‘20220725AB_MPM_18-9’. Each of the animal folders contain the following files: x_MT.nii,   x_T1.nii, x_PD.nii , x_t2.nii and x_mt2.nii.

SET MPM-PATH

-add MPM-path via hyperlink or go to mpm-folder and type “mpmlink” (this will set the paths of mpm)

-in Matlab command window type ‘mpm’ to open the mpm-GUI.

- Set Matlab’s current working Dir to the studies folder: Here: ‘F:\data8_MPM\MPM_agBrandt4_noANTx’.

SETUP and configuration

Hit the [setup]-button from the mpm-GUI.

Hit the [select]-button to set the destination path and select the study-folder

Hit [select]/[ordner auswählen].

Hit the [copy files]-button to copy 3 documents to the studies/mpm-folder. Hit [close]-button to close the GUI. The study folder should now contain a folder ‘mpm’ with the following documents:

-‘mpm_NIFTIparameters.xlsx’

-‘mpm_config.m’

-‘hmri_local_defaults_mouse.m’

1) ‘mpm_NIFTIparameters.xlsx’

The Excel-file looks as follows:

The 4th column specifies the file-names in standard space. Please do not change the names, rather change the names of the files in the ‘dat’-folder. Next add the parameters (flip angle, TR and TE) manually.

Save and close the excel-file.

2) ‘mpm_config’

The mpm_config.m file contains parameters that have to be adjusted:

mpm.MPM_path      ='F:\data7\MPM_mouse\hMRI-toolbox-0.2.4'   ;%MPM-toolbox path

-please specify the path of the hMRI-toolbox-0.2.4

mpm.SPM_path      ='F:\data7\MPM_mouse\spm12'                ;%SPM-toolbox path

-please specify the path of SPM12.

The mpm.hrmi_defaults   and mpm.NIFTI_parameters should be already defined (after the mpm-setup), and link to the files “hmri_local_defaults_mouse.m” and “mpm_NIFTIparameters.xlsx”:

mpm.hrmi_defaults         ='F:\data8_MPM\MPM_agBrandt4_noANTx\mpm\hmri_local_defaults_mouse.m';         %path of hMRI-configfile (m-file)

mpm.NIFTI_parameters ='F:\data8_MPM\ MPM_agBrandt4_noANTx\mpm\mpm_NIFTIparameters.xlsx';         %path of NIFTI-parameter settings (excelfile)

The mpm.t2w_preorient variable defines the rough orientation of the turborare-image (used for registration) to match the orientation of PD/T1/MT.

mpm.t2w_preorient=[1.5708 0 1.5708];  % preorientation (rotations) of turborare (t2) to match orientation with PD/T1/MT-images. HERE, the images are already in Standard space, so there is no need to change/modify this variable.

The mpm.multifactor   is set to 2000. This factor is multiplied to the PD/T1/MT-image values to increase the dynamic range.mpm.multifactor           = 2000;   % multiply images by this factor to widen dynamic range

PD-normalization

For normalization of the PD-image a reference image is necessary that represents a mask of the CSF/ventricles. A ventricle mask exists in the mpm-toolbox and the path is defined in mpm.PD_normalizeMask.

mpm.PD_normalizeMask      ='F:\mpm\resources\mask_ventricle.nii';         %reference-mask for normalization ('mask_ventricle.nii' or

'mask_water.nii' or use your own mask)

mpm.PD_normalizeFunction  =@mean                                       ;% function to aggregate the values within reference-mask

mpm.PD_normalizeOutputName='PD_normalized.nii'                         ;% resulting outputName

The mpm.PD_normalizeFunction defines how to aggregate the values within the reference mask (here as mean over values within the mask). The output name of the normalized PD-file is defined in mpm.PD_normalizeOutputName.

3) “hmri_local_defaults_mouse.m”

This m-file contains parameters for the hmri-toolbox.

Important here is that the parameter hmri_def.TPM links to the path of the tissue compartments (contained in the mpm-toolbox):

hmri_def.TPM='F:\mpm\resources\mouseTPM_mod.nii';

Loading the config-file

In the mpm-gui hit the [load config-file]-button and select the ‘mpm_config.m’-file. Hit [open]/[öffnen] to load the configuration.

When done, the mpm-GUI should display the path of the config-file right to the [load config-file] button.

Select animals for processing

From mpm-GUI hit the [select animals] button. In the animal-selection GUI navigate to the studies ‘dat’-folder (left listbox). Now select the two animals from the right listbox

When selected, the two animals disappear in the right listbox and reappear in the lower listbox. Hit [Done]-button.

The animal-listbox in the  mpm-GUI should now contain the selected animals for processing. Here two animals were selected).

RUN the hMRI-processing steps

From the hMRI-TBX processes check the [select all]-radio to run all processing steps.

Hit [RUN] to execute all selected processing steps for the selected animals.